http://delcom-products.com/productdetails.asp?PartNumber=900000

.NET C# Integration

- Obtain “USB HID MFC C#.NET Example Source Code” from:

http://delcom-products.com/downloads/HIDIOWINCS.zip

- Add the following files to your project:

"Hid.cs"

"HidDeclarations.cs"

"DeviceManagement.cs"

"DeviceManagementDeclarations.cs"

"FileIODeclarations.cs"

"DelcomHIDIO.cs"

- Create a DelcomHID object:

DelcomHID m_FootSw = new DelcomHID();

- Open CUSBHIDIO object & setup to poll device:

private System.Windows.Forms.Timer tmrChkFsw;

tmrChkFsw = new System.Windows.Forms.Timer(this.components);

tmrChkFsw.Interval = 150;	// 150mS

tmrChkFsw.Tick += new System.EventHandler(tmrChkFsw_Tick);

uint uPort = 0;

if ((m_FootSw.GetDevicesCount() == 1) && (m_FootSw.Open() == 0))

this.tmrChkFsw.Start();

- Poll footswitch:

//-------------------------------------------------------

//	tmrChkFsw_Tick

//	   Check Foot Switch

//-------------------------------------------------------

private void tmrChkFsw_Tick(object sender, EventArgs e)

{

UInt32 Port0 = 0;

if (m_FootSw.ReadPort0(ref Port0) == 0)

{

Port0 = ~Port0; // invert to ports values

if (((0x1 & (Port0 >> 1)) != 0) && BtnStopCapture.Enabled)

StopCaptureButton_Click(null, null);

else if (((0x1 & (Port0 >> 0)) != 0) && BtnStartCapture.Enabled)

StartCaptureButton_Click(null, null);

}

}

- Close the CUSBHIDIO object:

tmrChkFsw.Stop();

m_FootSw.Close();