http://delcom-products.com/productdetails.asp?PartNumber=900000

MFC C++ Integration

- Obtain “USB HID MFC C++ Example Source Code” from:

http://delcom-products.com/downloads/HIDIOWINC.zip

- Add the following files to your project:

"hidpi.h"

"hidsdi.h"

"hidusage.h"

"USBHIDIO.h"

"USBHIDIO.cpp"

- Create a CUSBHIDIO object:

#include "USBHIDIO\USBHIDIO.h"

CUSBHIDIO      m_FootSw;

- Open the CUSBHIDIO object & setup to poll device:

if (m_FootSw.IsOpen())

m_FootSw.CloseDevice();

if (m_FootSw.OpenDevice() == 0)

SetTimer(ID_FOOTSW_TIMER,150,NULL);

- Poll footswitch:

//-------------------------------------------------------

//	OnTimer

//	   Check Foot Switch

//-------------------------------------------------------

void OnTimer(UINT nIDEvent)

{

if (nIDEvent == ID_FOOTSW_TIMER)

{   // Check Foot Switch

if (m_FootSw.IsOpen())

{

unsigned int uiPorts;

if (m_FootSw.ReadPorts(&uiPorts) == 0)

{

uiPorts = ~uiPorts; // invert to ports values

if (0x1 & (uiPorts >> 1))

OnBnClickedSave();

else if (0x1 & (uiPorts >> 0) && !m_bCapturing)

OnBnClickedScan();

}

else MessageBox("ReadPorts Failed","Foot Switch"

" Error",MB_ICONERROR|MB_OK);

}

}

CDialog::OnTimer(nIDEvent);

}

- Close the CUSBHIDIO object:

m_FootSw.CloseDevice();